D200 Firmware Flash

Repair Tools and Steps:

- Install the Windows version of the flashing driver: DriverAssitant_v5.1.1

- Open the RKDevTool_v3.19_for_window flashing tool. Select "Upgrade Firmware", then choose the firmware file and import the "update" firmware into the tool.

Note: The compressed package contains two versions of the IMG file. Due to different device batches, the firmware may vary. You can first flash the v2.0.3 IMG file. If screen distortion occurs, then switch to the other IMG file labeled 1.7.40.

- Completely exit the Ulanzi Studio app on your computer. Connect the D200 using a USB cable and wait for it to power on. Press and hold Win+R, type cmd, and press Enter to open the Command Prompt. Type “adb reboot loader” and press Enter.

The device will enter the Starting-up state.

4、If the PC-based RK tool displays “A LOADER device has been detected,” it indicates the device has successfully connected to the computer.

5、Click "Upgrade" to start the firmware update. Wait until the download reaches 100% and the tool shows "Success". Then you can unplug the USB cable, plug it back in, and the fault will be resolved.

- 6、Download the Ulanzi Studio version from the link below. After launching it, do not upgrade Ulanzi Studio—skip the upgrade prompt. Use the V2.0.19 app version to push the update to your device and apply the firmware update.

- After the upgrade completes, go to Settings > Device to verify the firmware version is 2.0.3. This confirms the firmware upgrade succeeded, and the device is ready for normal use.

- https://cdn.ulanzistudio.com/UlanziDeck/2.0.19/Win_Ulanzi_Studio_V2.0.19.20251031.exe

- 7，After upgrading the device firmware to version 2.0.3, you can update the Ulanzi Studio app to the latest version.