Troubleshooting Steps for RK Solution-based D200 and D200H Models That Fail to Power On

Applicable only to the fault where the device cannot enter the normal interface after "Ulanzi Starting up" appears at the bottom right corner during startup.

Repair Tools and Steps:

1、Install the Windows version of the flashing driver: DriverAssitant_v5.1.1

- Open the RKDevTool_v3.19_for_window flashing tool. Select "Upgrade Firmware", then choose the firmware file and import the "update" firmware into the tool.

（Note:The compressed package contains two different versions of the IMG file. If the screen flickers upon startup after upgrading one of the firmware versions, it indicates an incorrect firmware selection. Please re-upgrade using the other firmware version.）

3、Remove the device's back cover and use tweezers to short-circuit pins 1 and 3 as shown in the images.

Note:

1.1 To prevent hardware short circuits, please ensure you follow the shorting instructions in the diagram and avoid touching other components.

1.2 Before powering on the Type-C interface, inspect the circuit board for cleanliness. Avoid allowing conductive metal foreign objects to fall onto the circuit board. Clean thoroughly before powering on.

- Keep the short-circuit connected (don’t let go of the tweezers) and plug in the USB cable to connect the PC and the device.

5、If the PC tool shows "A LOADER device found", it means the device is successfully connected to the PC. You can then release the tweezers. If the connection fails, the tweezers may not be short-circuited properly. Please try repeatedly—make sure to keep the short-circuit first, then plug in the USB cable.

6、Click "Upgrade" to start the firmware update. Wait until the download reaches 100% and the tool shows "Success". Then you can unplug the USB cable, plug it back in, and the fault will be resolved.

- 7、Download the Ulanzi Studio version from the link below. After launching it, do not upgrade Ulanzi Studio—skip the upgrade prompt. Use the V2.0.19 app version to push the update to your device and apply the firmware update.

- After the upgrade completes, go to Settings > Device to verify the firmware version is 2.0.3. This confirms the firmware upgrade succeeded, and the device is ready for normal use.

- https://cdn.ulanzistudio.com/UlanziDeck/2.0.19/Win_Ulanzi_Studio_V2.0.19.20251031.exe

- 8，After upgrading the device firmware to version 2.0.3, you can update the Ulanzi Studio app to the latest version.